SAMPLING  INSTRUCTIONS

OUR TIP - HAVE A WIPER HANDY JUST IN CASE YOU SPILL  ANY  OIL!

1)   Run unit to warm and circulate oil before taking sample.

- For convenience, take oil sample from the unit via the oil filler point

using syringe and plastic tubing provided.

With tube below oil level, but not touching the bottom of the oil

reservoir, pump syringe 2 -  3 times to expel air and then draw oil

sample into syringe.(For units with pumped and filtered lubrication

systems take sample before filter)

3)  Take sufficient oil out of unit to fill sample bottle to just below neck.

Seal bottle with plug, and then screw cap on securely.

4)  Fill in details on sample label provided, including your name and

contact details, unit make, model, type and your reference number if

any.

Please also give as much information as you can about  the oil in use

ie. oil manufacturer, oil type, viscosity  and time in service.

5)  Stick label around bottle.

6)  Finally, check cap is firmly screwed on to  bottle, put sample into

tube, secure end of tube using strong adhesive tape and post

++++++++++++++++++++++++++++++++++++++++++++++++++++++

The analysis report that you will receive from us will clearly explain the

condition of your oil sample, giving details of the physical condition of the

oil plus wear metals and other contaminants.

The report will start by advising you of the status of the sample :

SATISFACTORY :   EARLY WARNING  :    IMMEDIATE ACTION

SATISFACTORY - is what we generally expect to see. The results are within our recommended limits and your unit and oil are fit for further use.

EARLY WARNING  - will mean that we have identified a problem that may need investigation, either by further oil sampling, or we may recommend that you undertake some remedial work.

IMMEDIATE ACTION - has identified a major problem, one that requires immediate attention. Because failure of the  unit may be imminent, we will normally contact you by phone to save time, as well as  posting our report to you in the normal way.

We value your business and your comments. If you feel there is any way in which we could improve our service to you then please contact us.

THANK YOU FOR USING OUR SERVICE

The Oil Lab : Tel. 01254 677101 mobile 07951 255 198  email: info@theoillab.co.uk

The Oil Lab Ltd

15 Limefield, Blackburn, Lancashire, BB2 6PJ

Tel/Fax:- 01254 677101

www.theoillab.co.uk     info@theoillab.co.uk

Company Registration: 6371143     VAT Registration: 917 2556 16